Table of Contents

# DCIT 426 Telecommunications

## Topic 1: Course Introduction and Telecom Fundamentals

Syllabus line (Week 1): Evolution of telecommunications; signals, bandwidth, channels; analog vs digital; standards bodies (ITU, IEEE, IETF).

### Textbook Mapping

| Sub-topic | Where to Read | Notes |
| --- | --- | --- |
| System model, standards | Foruzan Ch. 1; Stallings Ch. 1 and Ch. 2 |  |
| Signals, bandwidth, time/frequency domain | Foruzan Ch. 3; Stallings Ch. 3 |  |
| Analog vs digital transmission | Stallings Ch. 3, Section 3.2 |  |
| History and evolution | Haykin & Moher Ch. 1 |  |

## 1.1 What “telecommunication” actually means

Short answer: communication over a distance, using electrical, electromagnetic or optical signals, where the message is carried by an artificial system rather than by your own voice.

The word breaks into two parts: - tele (Greek): far, at a distance - communicare (Latin): to share

So a person shouting across a room is communication. That same person calling from Legon to Kumasi is telecommunication, because a system had to convert the voice into a signal, carry it, and rebuild it at the other end.

### The five-block model of any telecom system

Every telecom system you will study this semester, from a telegraph in 1840 to a 5G base station in Achimota, fits this same picture. Memorise this diagram. Examiners love it.

+--------+     +-------------+      +-----------+      +----------+     +-------------+
  | SOURCE | --> | TRANSMITTER | ---> |  CHANNEL  | ---> | RECEIVER | --> | DESTINATION |
  +--------+     +-------------+      +-----------+      +----------+     +-------------+
   message         converts the         the medium        undoes what        the person
   (voice,         message into a       (copper, air,      the transmitter    or device
   text, video)    signal suited to     fibre, space)      did                that uses
                   the channel               ^                                the message
                                             |
                                        +---------+
                                        |  NOISE  |  <-- always present, never invited
                                        +---------+

Worked example (map the blocks yourself): you send a WhatsApp voice note from Madina to a friend in Tamale.

| Block | What plays that role |
| --- | --- |
| Source | Your voice, plus the phone microphone |
| Transmitter | Phone codec + modulator + antenna (turns bits into a radio wave) |
| Channel | Air to the MTN cell tower, then fibre backbone up north |
| Noise | Other phones on the same band, thermal noise in the amplifiers, rain fade |
| Receiver | Your friend’s antenna, demodulator, decoder |
| Destination | Their earpiece, and their brain |

Exam tip: when a question says “with the aid of a diagram, describe a general communication system,” this five-block diagram plus one sentence per block is the full-mark answer. Always draw noise entering at the channel, not at the source.

## 1.2 Evolution of telecommunications

You do not need every date. You need the pattern: each generation solved the bottleneck of the previous one.

### Global timeline

| Era | Milestone | Bottleneck it solved |
| --- | --- | --- |
| 1837 | Telegraph (Morse). Dots and dashes over a wire. | Distance. First time a message moved faster than a horse. |
| 1876 | Telephone (Bell). Continuous voice over a wire. | Telegraph needed trained operators and coded text. |
| 1895 | Radio (Marconi). Wireless telegraphy. | Wires cannot cross oceans cheaply or reach ships. |
| 1948 | Shannon publishes information theory; transistor invented. | No one knew the theoretical limit of a channel. |
| 1950s to 60s | Microwave relay, coaxial cable, satellite (Telstar, 1962). | Capacity for long-haul trunk routes. |
| 1970s | Optical fibre becomes practical (Corning low-loss glass); digital switching. | Copper capacity and noise accumulation. |
| 1980s to 90s | 1G analog cellular, then 2G GSM digital. Internet and TCP/IP spread. | Mobility, and incompatible networks. |
| 2000s | 3G data, broadband DSL, submarine fibre reaches West Africa. | Data, not just voice. |
| 2010s to now | 4G LTE, 5G NR, LEO satellite constellations, everything is IP. | Latency, density, and capacity per square kilometre. |

The one sentence that ties it together: telecommunications has moved from analog to digital, from fixed to mobile, from voice-centric to data-centric, and from separate networks per service to one converged IP network.

### Ghana timeline (very likely to appear in a “local context” question)

| Year | Event |
| --- | --- |
| 1881 | First telegraph lines under colonial administration, mainly Cape Coast and Accra |
| Post-1957 | Posts and Telecommunications (P&T) runs everything as a state monopoly |
| 1994 to 1995 | Sector reform. Ghana Telecom is separated out. Mobile licences issued. |
| 1996 | Ghana becomes one of the first African countries with commercial mobile and full internet connectivity |
| 2001 | SAT-3/WASC submarine cable lands at Accra, replacing satellite-only international links |
| 2008 | Vodafone acquires 70% of Ghana Telecom |
| 2010 to 2012 | MainOne, Glo-1 and ACE submarine cables land, prices per Mbps collapse |
| 2021 | Government acquires AirtelTigo, later rebranded AT Ghana |
| 2023 to 2024 | Telecel Group acquires Vodafone Ghana, rebranded Telecel Ghana in March 2024 |
| 2024 to 2026 | 5G introduced through a shared wholesale model (Next Gen InfraCo). The NCA later removed NGIC’s exclusivity and, in July 2026, opened a competitive request for applications for spectrum in the 700 MHz, 2.3 GHz and 3 GHz bands. |

Ghana today: three mobile network operators (MTN, Telecel, AT), with MTN designated by the NCA as having Significant Market Power, which means it faces stricter price and interconnection rules than the others. Most subscribers are still on 3G and 4G, not 5G.

Why the lecturer cares: this is the raw material for exam questions like “Discuss how liberalisation of the telecom sector affected service delivery in Ghana” or “Explain why submarine cable landings reduced internet cost in Ghana.” Learn the causal story, not just the dates.

## 1.3 Signals

Short answer: a signal is a physical quantity (usually voltage, current, or light intensity) that varies with time and carries information.

### Analog vs digital signals

ANALOG SIGNAL                          DIGITAL SIGNAL
(infinitely many values)               (finite set of values, here 2)

  amplitude                              amplitude
     ^      __                              ^   ___     ___
     |    /    \      __                    |  |   |   |   |
     |  /       \   /   \                   |  |   |   |   |
     |_/         \_/     \___               |__|   |___|   |____
     +----------------------> t             +--------------------> t
     smooth, continuous                     abrupt, discrete steps
     e.g. human voice on a landline         e.g. 1 0 1 1 0 on a LAN cable

|  | Analog | Digital |
| --- | --- | --- |
| Values | Continuous, infinite | Discrete, finite (usually 0 and 1) |
| Example | Human speech, FM radio wave, old vinyl | Computer data, CD audio, Ethernet |
| Noise | Noise blends into the signal and cannot be removed | Noise can be rejected as long as 0 and 1 are still distinguishable |

Do not confuse two different pairs. This is the single most common mix-up in this course. 1. Analog vs digital data (the information itself: voice is analog data, a text file is digital data) 2. Analog vs digital signal (how it is physically represented on the medium)

All four combinations exist:

| Data | Signal | Real example |
| --- | --- | --- |
| Analog data | Analog signal | FM radio broadcast |
| Analog data | Digital signal | Your voice on a mobile phone (codec digitises it) |
| Digital data | Analog signal | Old dial-up modem, or Wi-Fi (bits ride on a radio carrier) |
| Digital data | Digital signal | Ethernet cable in the lab |

### The sine wave and its three parameters

The sine wave is the atom of signal theory. Everything else is built from sine waves.

s(t) = A sin(2πft + φ)

A |     ___                       Peak amplitude A = height
          |   /     \                     Frequency f     = cycles per second (Hz)
        0 |--/-------\-------/---->  t    Phase φ         = starting position
          | /         \     /
       -A |/           \___/
          |<--- T --->|                   Period T = time for one cycle

| Parameter | Meaning | Unit | Change it and… |
| --- | --- | --- | --- |
| Amplitude (A) | Strength or height of the signal | volts | the sound gets louder |
| Frequency (f) | How many cycles per second | hertz (Hz) | the pitch gets higher |
| Phase (φ) | Where in the cycle it starts | degrees or radians | it shifts left or right in time |

Two formulas you must know cold:

T = 1 / f            period and frequency are reciprocals
λ = c / f            wavelength, where c = 3 x 10^8 m/s in free space

Quick numeric check. A signal has f = 1 kHz. - T = 1/1000 = 1 ms - λ = (3 x 10^8) / 1000 = 300,000 m. That is why very low frequencies need enormous antennas.

A GSM signal at 900 MHz: λ = (3 x 10^8)/(900 x 10^6) = 0.333 m. A practical antenna is a fraction of that, roughly 8 cm. This is the reason higher frequency means smaller antennas, which is why 5G mmWave devices can pack many antennas into one phone.

### Time domain vs frequency domain

Same signal, two ways of drawing it. This idea underpins Week 2, 4 and 5, so get it now.

TIME DOMAIN                             FREQUENCY DOMAIN
"what the signal looks like"            "what ingredients it is made of"

 amp ^   /\    /\    /\                  amp ^
     |  /  \  /  \  /  \                     |     |
     | /    \/    \/    \                    |     |
     +-------------------> time              +-----+------------> frequency
                                                  1 kHz
   a 1 kHz sine wave                        a single line at 1 kHz

A composite signal (any real-world signal, like speech) is a sum of many sine waves of different frequencies, amplitudes and phases. Fourier analysis is the mathematics that decomposes it. In the frequency domain a composite signal shows up as several lines or a continuous band, called its spectrum.

Composite signal made of 1 kHz + 3 kHz + 5 kHz (a square-ish wave)

 FREQUENCY DOMAIN
   amp ^
       |  |
       |  |    |
       |  |    |    |
       +--+----+----+-------> f (kHz)
          1    3    5

   Spectrum = the set of frequencies present
   Bandwidth = 5 - 1 = 4 kHz

Analogy: a plate of jollof is the time domain, what you actually see and eat. The frequency domain is the recipe list: rice, tomato, oil, spice, in specific quantities. Fourier analysis is the chef who tastes the dish and writes out the recipe.

## 1.4 Bandwidth

Short answer: bandwidth is the width of a band of frequencies, measured in hertz. It is NOT the same as data rate, which is measured in bits per second.

This distinction is examined nearly every year.

| Term | Symbol | Unit | Meaning |
| --- | --- | --- | --- |
| Bandwidth (analog sense) | B | Hz | highest frequency minus lowest frequency |
| Data rate (digital sense) | C or R | bps | how many bits pass per second |
| Throughput | - | bps | the data rate actually achieved in practice |

Formula: B = f_highest - f_lowest

### Worked example: the telephone voice channel

Human speech has energy roughly from 100 Hz to 7 kHz, but intelligible speech survives on far less. The telephone system therefore passes only 300 Hz to 3400 Hz.

|<------------- 4000 Hz channel slot ------------->|
   |guard|<---- 3100 Hz usable voice band ---->|guard |
   0    300                                   3400   4000  Hz

- Signal bandwidth = 3400 - 300 = 3100 Hz

- Allocated channel bandwidth = 4 kHz, the extra 900 Hz being guard bands to stop neighbouring channels bleeding into each other

Why bandwidth matters: more bandwidth means more room to carry information. Nyquist and Shannon (Week 2) turn this into hard numbers, but the intuition is already available to you: a wider pipe carries more water per second.

Analogy for the road: bandwidth is the number of lanes on the Accra to Tema motorway. Data rate is how many cars actually arrive per minute. More lanes usually means more cars, but not if traffic is bad (that is throughput), and never more than the physics allows (that is Shannon capacity).

### Baseband vs broadband

|  | Baseband | Broadband |
| --- | --- | --- |
| Meaning | Signal sent at its original frequencies | Signal shifted onto a higher-frequency carrier |
| Channel usage | Whole medium used for one signal | Medium divided into several frequency channels |
| Example | Ethernet over UTP | Cable TV, or FM radio, or Wi-Fi |

## 1.5 Channels

Short answer: the channel is the path the signal travels between transmitter and receiver, together with everything that path does to the signal.

### Guided vs unguided

GUIDED (wired)                             UNGUIDED (wireless)
signal is confined inside a medium         signal spreads through space

 [TX]===[twisted pair]===[RX]                [TX] ))) ))) ))) [RX]
 [TX]===[coaxial cable]===[RX]                     radio waves,
 [TX]===[optical fibre]===[RX]                     microwave, infrared

|  | Guided | Unguided |
| --- | --- | --- |
| Medium | Copper pair, coax, fibre | Air, vacuum, water |
| Direction control | Physical path decides it | Antenna pattern decides it |
| Security | Harder to intercept | Easier to intercept |
| Mobility | None | Full |
| Ghana example | ECG poles carrying fibre along the N1 | MTN 4G on 800 MHz in Adenta |

Week 3 covers each medium properly. For now just know the categories.

### Transmission modes: simplex, half-duplex, full-duplex

SIMPLEX             A ------------------>  B      one direction only
                    e.g. radio broadcast, keyboard to computer

HALF DUPLEX         A <-----------------> B      both directions,
                    but only one at a time       e.g. walkie-talkie,
                                                 "over"

FULL DUPLEX         A ==================> B      both directions
                    A <================== B      simultaneously
                                                 e.g. phone call

| Mode | Both directions? | At the same time? | Example |
| --- | --- | --- | --- |
| Simplex | No | - | TV broadcast, GPS receiver |
| Half-duplex | Yes | No | Walkie-talkie, old CSMA/CD Ethernet hub |
| Full-duplex | Yes | Yes | Telephone call, modern switched Ethernet |

Common exam trap: a mobile phone call is full-duplex even though there is only one antenna. It achieves this using FDD (separate uplink and downlink frequencies) or TDD (rapid alternation in time). You will meet both in Week 5 and Week 9.

### What the channel does to your signal

Every channel degrades the signal. The three main effects (detailed in Week 2):

| Impairment | What it does | Everyday sign |
| --- | --- | --- |
| Attenuation | Signal loses strength with distance | Voice gets faint on a long line |
| Distortion | Different frequencies travel at different speeds, so the shape changes | Muffled or smeared audio |
| Noise | Unwanted energy added to the signal | Hiss, crackle, static |

## 1.6 Analog vs digital transmission, and why digital won

This is a classic essay question. Learn the argument, not just the list.

### The key mechanism: repeaters vs amplifiers

ANALOG LINK
             noise         noise         noise
               |             |             |
  [TX]---------v-----[AMP]---v----[AMP]----v----[RX]
  Signal is amplified. But the amplifier cannot tell signal from
  noise, so it amplifies BOTH. Noise accumulates and never leaves.
  After many hops the signal is unusable.

DIGITAL LINK
             noise         noise         noise
               |             |             |
  [TX]---------v---[REGEN]---v---[REGEN]---v----[RX]
  A regenerative repeater DECIDES whether it sees a 0 or a 1,
  then transmits a brand new clean pulse. Noise is discarded
  at every hop. The signal at the far end is as clean as at hop 1.

That single difference is the heart of why the world went digital. Everything else follows from it.

### Full advantages of digital transmission

| Advantage | Explanation |
| --- | --- |
| Noise immunity | Regeneration removes accumulated noise, as shown above |
| Error detection and correction | You can add parity, checksums, CRC, FEC to bits. You cannot do that to an analog waveform. (Week 6) |
| Encryption | Bits can be scrambled with a key. Analog scrambling is weak. (Week 13) |
| Easy multiplexing | Time-division multiplexing of bit streams is simple and cheap. (Week 5) |
| Integration of services | Voice, video, text and data all become “just bits,” so one network carries everything. This is convergence. |
| Cheap hardware | Digital circuits ride the cost curve of VLSI and software. Analog circuits do not. |
| Storage and processing | Bits can be buffered, compressed, and processed by computers |

### Disadvantages of digital

- Digitising analog data needs more bandwidth. A 4 kHz voice channel becomes 64 kbps PCM, which needs far more than 4 kHz of raw spectrum unless clever modulation is used.

- Quantisation error is introduced when converting analog to digital, and it can never be undone.

- Requires synchronisation between sender and receiver, which analog does not.

Scenario to remember: compare an old analog trunk line from Accra to Bolgatanga with today’s fibre link. On the analog line each amplifier along the way added hiss, and by Bolga the caller sounded like they were in a storm. On the fibre link, every regenerator rebuilds clean bits, so the voice at Bolga is bit-for-bit identical to the voice at Accra. Distance stops degrading quality.

## 1.7 Standards and standards bodies

### Why standards exist

Short answer: so that equipment from different manufacturers in different countries can interoperate.

Without standards, an MTN SIM would not work on a Telecel tower, a Huawei router would not talk to a Cisco switch, and a call from Accra to London would stop at the border. Standards give you: - Interoperability across vendors - Competition, which lowers prices, because buyers are not locked to one vendor - Economies of scale in manufacturing - A common technical language for engineers and regulators

### De jure vs de facto

|  | De jure (“by law”) | De facto (“by fact”) |
| --- | --- | --- |
| Origin | Formally approved by a recognised body | Became standard through widespread adoption |
| Example | ITU-T G.711, IEEE 802.11 | TCP/IP originally, PDF, HTTP cookies |
| Two sub-types of de facto | Proprietary/closed (owned by one company, e.g. early Cisco IGRP) and open (published freely) |  |

### The bodies you must know

| Body | Full name | Scope | Output is called | Examples |
| --- | --- | --- | --- | --- |
| ITU | International Telecommunication Union | UN agency. Global telecom coordination. | Recommendations | See below |
| IEEE | Institute of Electrical and Electronics Engineers | LAN, MAN, physical and data-link layers | Standards (numbered) | 802.3 Ethernet, 802.11 Wi-Fi, 802.15 Bluetooth/Zigbee, 802.16 WiMAX |
| IETF | Internet Engineering Task Force | Internet protocols, layer 3 and above | RFCs (Request for Comments) | RFC 791 IP, RFC 793 TCP, RFC 2616/7230 HTTP |
| ISO | International Organization for Standardization | Broad, all industries | ISO standards | ISO/IEC 7498, the OSI model |
| 3GPP | 3rd Generation Partnership Project | Mobile: GSM, UMTS, LTE, 5G NR | Releases and TS documents | Release 15 = first 5G |
| ETSI | European Telecommunications Standards Institute | European telecom, founded GSM | ETSI standards | GSM |
| ANSI | American National Standards Institute | US national coordination | ANSI standards | ANSI T1 |
| IANA / ICANN | Internet Assigned Numbers Authority / ICANN | IP address and domain name allocation | Registries | .gh domain delegation |
| NCA | National Communications Authority | Ghana’s national regulator | Licences, directives, type approval | Spectrum licensing, SIM registration, quality-of-service standards |

### The three ITU sectors (frequently asked)

ITU
                           |
       +-------------------+-------------------+
       |                   |                   |
     ITU-T               ITU-R               ITU-D
  Telecommunication    Radiocommunication   Development
  Standardization
  wireline, protocols,  spectrum allocation,  helping developing
  codecs, signalling    satellite orbits,     countries build
  e.g. G.711, H.264,    World Radio           telecom capacity
  V.90, X.25            Conferences (WRC)

Memory hook: T for Telephone and Technical standards, R for Radio and spectrum, D for Development.

Ghana connection: the ITU-R decides globally which frequency bands are for mobile. The NCA then decides who in Ghana may use those bands, and on what terms. That is exactly why the NCA’s July 2026 request for applications names the 700 MHz, 2.3 GHz and 3 GHz bands. Those band designations came from ITU-R decisions, and the NCA is turning them into national licences. This is the layered relationship between international standards and national regulation, which is a strong point to make in Week 13’s regulation essay.

## 1.8 Formula sheet for Topic 1

T = 1/f                     period from frequency
f = 1/T                     frequency from period
λ = c/f                     wavelength, c = 3 x 10^8 m/s
B = f_high - f_low          bandwidth of a composite signal
dB = 10 log10 (P2/P1)       power gain or loss in decibels

Decibel worked example. A signal enters a cable at 10 mW and leaves at 5 mW. dB = 10 log10 (5/10) = 10 x (-0.301) = -3.01 dB A loss of about 3 dB means the power halved. Learn this pair: 3 dB = double or half the power. 10 dB = ten times.

## 1.9 Traps that cost marks

- Bandwidth is in Hz, data rate is in bps. Never write “bandwidth of 10 Mbps” in this exam, even though people say it casually.

- Analog data is not the same as an analog signal. Check which one the question asks about.

- Noise enters at the channel, not at the source. Draw it that way.

- Amplifier ≠ repeater. Amplifiers boost signal and noise together. Regenerative repeaters rebuild clean digital pulses.

- ITU-T vs ITU-R. Anything about spectrum or satellite orbits is ITU-R. Anything about codecs, protocols or wireline is ITU-T.

- IEEE handles layers 1 and 2. IETF handles layer 3 upward. Wi-Fi is IEEE. IP is IETF.

- Full-duplex does not require two cables. It requires two separated channels, which can be by frequency or by time.

## 1.10 Glossary

| Term | Definition in one line |
| --- | --- |
| Signal | A physical quantity varying with time that carries information |
| Amplitude | The strength or height of a signal |
| Frequency | Cycles per second, measured in hertz |
| Period | Time for one complete cycle, the reciprocal of frequency |
| Phase | The position of the waveform relative to time zero |
| Spectrum | The set of frequencies contained in a signal |
| Bandwidth | The width of that set, in hertz |
| Data rate | Bits transferred per second |
| Throughput | The data rate actually achieved |
| Channel | The path plus medium between transmitter and receiver |
| Guided medium | A medium that physically confines the signal, such as fibre |
| Unguided medium | Free space propagation, such as radio |
| Attenuation | Loss of signal strength over distance |
| Distortion | Change in signal shape caused by frequency-dependent delay |
| Noise | Unwanted energy added to a signal |
| Baseband | Transmission at the signal’s original frequencies |
| Broadband | Transmission after shifting the signal onto a carrier |
| Regenerative repeater | A device that decides the bit value and retransmits a clean pulse |
| Convergence | Carrying voice, video and data over one common network |
| De jure standard | A standard formally approved by a recognised body |
| De facto standard | A standard established by widespread use |

# Objective questions (Topic 1)

Attempt them all before checking the answers at the bottom.

Q1. In a general communication system, noise is normally shown as entering at the: A. Source B. Transmitter C. Channel D. Destination

Q2. A signal has a period of 4 ms. Its frequency is: A. 4 Hz B. 250 Hz C. 400 Hz D. 4 kHz

Q3. The bandwidth of a composite signal that contains frequencies from 500 Hz to 4500 Hz is: A. 500 Hz B. 4000 Hz C. 4500 Hz D. 5000 Hz

Q4. Which of the following is measured in hertz? A. Throughput B. Data rate C. Bandwidth D. Bit error rate

Q5. Human voice carried over a normal mobile phone network is an example of: A. Analog data on an analog signal B. Analog data on a digital signal C. Digital data on an analog signal D. Digital data on a digital signal

Q6. The main advantage of a regenerative repeater over an amplifier is that it: A. Increases the bandwidth of the channel B. Removes accumulated noise by rebuilding the pulse C. Reduces the propagation delay D. Converts digital signals to analog

Q7. A walkie-talkie operates in which transmission mode? A. Simplex B. Half-duplex C. Full-duplex D. Multiplex

Q8. Which body publishes documents known as RFCs? A. ITU B. IEEE C. IETF D. ISO

Q9. Which ITU sector is responsible for allocating radio frequency spectrum? A. ITU-T B. ITU-R C. ITU-D D. ITU-S

Q10. IEEE 802.11 is the standard for: A. Ethernet B. Wireless LAN C. Bluetooth D. Token Ring

Q11. The wavelength of a 1.8 GHz GSM signal in free space is approximately: A. 1.67 cm B. 16.7 cm C. 1.67 m D. 16.7 m

Q12. The standard voice channel in the telephone network is allocated: A. 3.1 kHz B. 4 kHz C. 8 kHz D. 64 kHz

Q13. Which of the following is a de facto standard? A. ITU-T G.711 B. IEEE 802.3 C. TCP/IP in its early years D. ISO 9001

Q14. Attenuation refers to: A. Addition of unwanted energy to a signal B. Loss of signal strength with distance C. Change in signal shape due to unequal delays D. Reflection of a signal from an obstacle

Q15. Which regulator issues telecommunications licences in Ghana? A. ITU B. NCA C. GIFEC D. Data Protection Commission

Q16. A power loss of 3 dB corresponds to approximately: A. One tenth of the power B. One half of the power C. Double the power D. Three times the power

Q17. Which of the following is NOT an advantage of digital transmission? A. Easier encryption B. Error detection and correction C. Lower bandwidth requirement for voice D. Noise immunity through regeneration

Q18. The mathematical technique that decomposes a composite signal into its constituent sine waves is called: A. Modulation B. Fourier analysis C. Quantisation D. Multiplexing

Q19. 3GPP is the body responsible for standardising: A. Wireless LANs B. Mobile cellular systems such as LTE and 5G NR C. Optical fibre connectors D. Internet routing protocols

Q20. Guided media differ from unguided media mainly in that guided media: A. Are always faster B. Physically confine the signal within a medium C. Cannot carry digital signals D. Require no transmitter

## Answers with explanations

| Q | Ans | Why |
| --- | --- | --- |
| 1 | C | Noise is a property of the channel and the receiver front end, not of the message |
| 2 | B | f = 1/T = 1/0.004 = 250 Hz |
| 3 | B | 4500 - 500 = 4000 Hz |
| 4 | C | Bandwidth is a width of frequencies. The other three are in bits per second or a ratio |
| 5 | B | Voice is analog data. The phone digitises it with a codec, so it travels as a digital signal |
| 6 | B | It makes a hard decision on the bit and transmits a fresh pulse, discarding noise |
| 7 | B | Both parties can speak, but not at the same time, hence “over” |
| 8 | C | IETF documents are RFCs. ITU issues Recommendations, IEEE issues numbered Standards |
| 9 | B | R for Radiocommunication, which manages spectrum and satellite orbits |
| 10 | B | 802.3 is Ethernet, 802.15 is Bluetooth, 802.11 is Wi-Fi |
| 11 | B | λ = 3x10^8 / 1.8x10^9 = 0.167 m = 16.7 cm |
| 12 | B | Usable band is 300 to 3400 Hz (3.1 kHz), allocated slot is 4 kHz including guard bands |
| 13 | C | TCP/IP spread through use before formal endorsement. The others were formally approved |
| 14 | B | A is noise, C is distortion, D is reflection |
| 15 | B | The National Communications Authority regulates and licenses the sector |
| 16 | B | 10 log10(0.5) = -3.01 dB, so 3 dB down is half the power |
| 17 | C | Digitised voice needs MORE raw bandwidth, not less. That is a disadvantage |
| 18 | B | Fourier analysis moves a signal from the time domain to the frequency domain |
| 19 | B | 3GPP produces the Releases that define GSM through 5G NR |
| 20 | B | Confinement within a physical medium is the defining property |

# Exam-style written questions with model answers

W1. With the aid of a labelled diagram, describe the components of a general telecommunication system. (6 marks)

Model answer: Draw the five-block diagram with noise entering at the channel. Then: the source produces the message. The transmitter converts the message into a signal suited to the channel, typically by encoding and modulating it. The channel is the medium that carries the signal and is the point at which noise and impairments are introduced. The receiver performs the inverse of the transmitter, demodulating and decoding the signal. The destination consumes the recovered message. One mark per correctly labelled block, one for showing noise at the channel.

W2. Distinguish between bandwidth and data rate, giving units and one example of each. (4 marks)

Model answer: Bandwidth is the width of the range of frequencies a signal occupies or a channel can pass, measured in hertz. A telephone voice channel has a bandwidth of about 3.1 kHz. Data rate is the number of bits transferred per second, measured in bits per second. A digitised voice channel using PCM has a data rate of 64 kbps. They are related, since a wider bandwidth permits a higher data rate, and Nyquist and Shannon quantify this relationship, but they are different quantities with different units.

W3. Explain, with reference to noise, why digital transmission has largely replaced analog transmission in long-distance networks. (6 marks)

Model answer: In an analog link, signal strength is restored by amplifiers. An amplifier cannot distinguish signal from noise, so any noise picked up on the preceding section is amplified along with the signal. Noise therefore accumulates cumulatively along the route, and after many sections the signal-to-noise ratio becomes too poor for the message to be recovered. In a digital link, regenerative repeaters make a decision on whether each received pulse represents a 0 or a 1 and then transmit an entirely new, clean pulse. Provided the noise has not been severe enough to cause a wrong decision, the noise is discarded at every repeater rather than accumulated. The received signal at the far end is therefore effectively identical to the transmitted one regardless of distance. Digital transmission additionally permits error detection and correction, encryption, and simple time-division multiplexing, and it allows voice, video and data to be carried over a single converged network.

W4. Differentiate between de jure and de facto standards, and name two standards bodies relevant to telecommunications, stating the scope of each. (6 marks)

Model answer: A de jure standard is one formally developed and approved by a recognised standards organisation. A de facto standard is one that has become widely accepted through market adoption without such formal approval, and it may be proprietary or open. Two bodies: the ITU, a United Nations agency whose ITU-T sector standardises wireline telecommunication protocols and codecs while ITU-R allocates radio spectrum and satellite orbital positions; and the IETF, which develops internet protocols at the network layer and above and publishes them as RFCs, for example IP in RFC 791.

W5. Discuss how the evolution of Ghana’s telecommunications sector since 1994 has affected service delivery and cost. (8 marks)

Points to build the answer on: the state monopoly under P&T gave low penetration and long waiting lists for fixed lines. Liberalisation from 1994 to 1995 separated Ghana Telecom and licensed private mobile operators, introducing competition. Mobile technology bypassed the cost of laying fixed copper to every home, so penetration rose rapidly and now exceeds 100% by SIM count. The arrival of submarine cables (SAT-3 in 2001, then MainOne, Glo-1 and ACE from 2010 to 2012) replaced expensive satellite backhaul, greatly reducing wholesale bandwidth cost per Mbps and enabling affordable retail data. Consolidation followed, with Vodafone becoming Telecel and AirtelTigo becoming state-owned AT, leaving three MNOs. Because MTN holds a very large share, the NCA has designated it as having Significant Market Power and applies asymmetric price regulation to protect competition. Most recently 5G has been introduced through a shared wholesale infrastructure model, and in 2026 the NCA opened competitive spectrum licensing in the 700 MHz, 2.3 GHz and 3 GHz bands. Conclude with the trade-off: competition and cable capacity lowered prices and expanded access, but rural coverage, quality of service and affordability of devices remain the constraints.

W6. A sinusoidal signal has a peak amplitude of 5 V and completes 2000 cycles in one second. Determine its frequency, period, and wavelength in free space. (4 marks)

Model answer: - f = 2000 Hz - T = 1/f = 1/2000 = 0.5 ms - λ = c/f = (3 x 10^8)/2000 = 150,000 m = 150 km State the general expression as s(t) = 5 sin(2π x 2000 t + φ).

## What to do before moving on

You should be able to, without notes: 1. Draw and label the five-block system diagram including noise 2. State three parameters of a sine wave and the relationship between period and frequency 3. Explain the difference between bandwidth and data rate with correct units 4. Give all four combinations of analog/digital data and signal with an example each 5. Explain the amplifier versus regenerative repeater argument 6. Name ITU, IEEE, IETF and NCA and say precisely what each is responsible for

Once all six are solid, move to Topic 2, which turns the intuition about bandwidth into the Nyquist and Shannon equations.